CCNA [Network Troubleshooting - A1]

Section A: Scenario-Based Concept Application

1. Deep Packet Inspection (DPI)

Deep Packet Inspection (DPI) is used to analyze network traffic deeply at Network and Transport layers.

It helps startup companies identify:

- Slow applications

- Traffic bottlenecks

- High bandwidth usage

- Suspicious traffic

Example

Even if internet bandwidth is high, one application like video streaming or large downloads may consume most bandwidth.

DPI checks:

- Source IP

- Destination IP

- Port numbers

- Protocol type

This helps network administrators find which traffic is causing slow internet.

2. DHCP and DNS Roles

Dynamic Host Configuration Protocol automatically assigns:

- IP address

- Subnet mask

- Gateway

- DNS server

Domain Name System converts domain names into IP addresses.

Scenario

If devices receive IP addresses successfully but websites do not open:

- DHCP is working properly

- DNS is failing

Result

Users can ping IP addresses but cannot open websites using names like google.com.

3. Inter-VLAN Routing and Static Routes

VLANs separate departments into different networks.

Example:

- VLAN 10 → HR

- VLAN 20 → Accounts

Devices in different VLANs cannot communicate directly.

Inter-VLAN Routing

Inter-VLAN routing allows communication between VLANs using:

- Router

- Layer 3 switch

Static Routes

Static routes tell router where packets should go.

Benefits:

- Better control

- Simple routing

- Secure communication between departments

4. ICMP vs HTTP/HTTPS Performance

Internet Control Message Protocol is used for ping testing.

Hypertext Transfer Protocol and HTTPS are used for website access.

Difference

Ping uses small packets and checks only connectivity.

HTTP/HTTPS involves:

- Web server processing

- Encryption

- Browser rendering

- Database communication

Result

Ping may respond fast while website loads slowly because web services are overloaded.

5. Firewall ACLs and Placement

Firewall ACLs control which traffic is allowed or denied.

Example

Management department should not be accessible by general staff.

ACL rules can:

- Allow authorized users

- Block unauthorized access

- Improve network security

Firewall Placement

Firewall should be placed:

- Between departments

- Between internal network and internet

Benefits:

- Traffic filtering

- Security monitoring

- Reduced unauthorized access

Section B: Practical Application

Task 1: VLSM Subnet Design

Network: 192.168.10.0/24

Requirement:

- 3 departments

- 15 hosts each

For 15 hosts, subnet mask required:

/27 = 255.255.255.224

Subnets

| Department | Network ID | Host Range | Broadcast |
| --- | --- | --- | --- |
| VLAN 10 | 192.168.10.0/27 | 192.168.10.1 - 192.168.10.30 | 192.168.10.31 |
| VLAN 20 | 192.168.10.32/27 | 192.168.10.33 - 192.168.10.62 | 192.168.10.63 |
| VLAN 30 | 192.168.10.64/27 | 192.168.10.65 - 192.168.10.94 | 192.168.10.95 |

Task 2: Configure VLANs on Cisco Switch

Create VLANs:

enable

configure terminal

vlan 10

name HR

vlan 20

name SALES

vlan 30

name ADMIN

Assign interfaces:

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 20

interface fa0/3

switchport mode access

switchport access vlan 30

Task 3: DHCP Pool Configuration

Configure DHCP for VLAN 10:

ip dhcp pool VLAN10

network 192.168.10.0 255.255.255.224

default-router 192.168.10.1

dns-server 8.8.8.8

This allows devices to get:

- IP address

- Gateway

- DNS automatically

Task 4: Wireshark Packet Capture

Steps:

- Start packet capture

- Open website

- Filter DNS traffic:

- Dns

- Filter HTTP traffic:

- http

Identify

DNS Query:

- source IP

- Destination DNS server IP

HTTP GET:

- Client IP

- Web server IP

- Requested website

Success Criteria

- Correct subnet mask for 15 hosts

- VLAN communication working

- Devices receive DHCP configuration

- Successful ping between VLANs

- Wireshark capture successful

Section C: Integrated Case Study

1. OSI Layers Most Likely Causing Issue

Most likely layers:

- Physical Layer

- Data Link Layer

Reason

- Packet loss indicates cable or switch issue

- Flapping connectivity often happens because of switching problems

2. Bottom-Up Troubleshooting Process

Step 1

Check cables and ports.

Step 2

Check switch interface status.

Step 3

Check VLAN configuration.

Step 4

Test connectivity using ping and traceroute.

This process starts from lower OSI layers and moves upward.

3. CLI Commands for Troubleshooting

Check interface status:

show interfaces

Check MAC address table:

show mac address-table

Check spanning-tree status:

show spanning-tree

These commands help identify:

- Port errors

- Broadcast storms

- Switching loops

4. Switching Loop and Prevention

Spanning Tree Protocol prevents switching loops.

Why It Failed

Possible reasons:

- STP disabled

- Wrong switch configuration

- Faulty switch ports

Result

Loop causes:

- Broadcast storm

- High latency

- Packet loss

- Network instability

5. Long-Term Configuration Change

To prevent one department issue affecting others:

Solutions

- Separate VLANs properly

- Enable STP

- Configure port security

- Use managed switches

- Apply ACLs between departments

Benefits:

- Better security

- Fault isolation

- Stable network performance